Laser points’ clouds – Cracow example

Content

Laser points’ clouds – Cracow example	1

Content	1

Name:	1

Data:	1

1. Aim of the laboratory	1

2. Data	1

3. Read ALS data	1

4. Cross section creation	2

5. Building digitisation	2

- Name:

- Data:

- Aim of the laboratory

The main aim of the laboratory is to be familiar with the ALS data

- Data

- points’ cloud – krk_00000nr.bin (nr  - student’s number)

– krk_skaning.dgn

- ortofotomaps – 969.tif, 1019.tif, 1020.tif, 1070.tif, 1071.tif

- Read ALS data

1. Run  MicroStation v8, with  krk_skaning.dgn.

2. Close AccuDraw.

3. Run MDL – tscan, Narzędzia>Aplikacje MDL>…

4. Read classified points’ cloud:

(main window tscan) >File > Read points>- (*.bin)> Add>Done

5. Tools > Show statistics

6. View>Medium Dialog

7. Del others areas, leave only own area.

- Cross section creation

1. Create points on the cross section each 5 m,

4. Create cross section using Tscan tool.

5. Display in View 2.

6. Read height in 15 points Identyfy from Tscan

7. Add point in characteristic point on embankments etc.

-  punt a new point MSt Umieść aktywny punkt

- Project point on reference line: MSt Rzutuj aktywny punk na element. Show line next projected point. Then measure the horizontal distance to the nearest Mierz odległość.

8. Draw DSM.

- Building digitisation

1. Digitise building Tscan Mouse Point Adjustment , on layer: building, MSt Umieść wielokąt

2. On cross section (Tscan) verify if the roof edges are on the correct height

3. Projection of the building verify on ortophoto. MSt: Plik>Menadżer plików rastrowych …Plik>Dołącz>wybrać plik *.tif>OK. Interactive snapping to the frame.