Satellite image – enhancement of multispectral images – statistical evaluation

Content

Satellite image – enhancement of multispectral images – statistical evaluation	1

Content	1

Name:	1

Data:	1

1. Aim of the laboratory	1

2. Data	1

3. Statistical evaluation	1

4. Results	3

4.1	Landsat_Dobczyce_1997	3

4.2	EO1 ALI (9 kanałów)*:	3

5. Summary	4

- Name:

- Data:

- Aim of the laboratory

The main aim of the laboratory is to be familiar with the statistical image evaluation taking into account information content

- Data

- Landsat_Dobczyce_1997

- EO1 ALI

- Statistical evaluation

MOIK(KBxyz) = |rxy| + |rxz| + |ryz|

OIF(KBxyz) = (x + y + z) / MOIK(KBxyz)

| CC | MOIK | rank | OIK | rank | rank of visual interpretation |
| --- | --- | --- | --- | --- | --- |
| 123 |  |  |  |  |  |
| 234 |  |  |  |  |  |
| 354 |  |  |  |  |  |
| 247 |  |  |  |  |  |
| 457 |  |  |  |  |  |

- Results

  - Landsat_Dobczyce_1997

Estimate the compositions: CC 123, 234, 354, 247 i 457

1. perform evaluation basing on the OIF i MOIK parameters

2. is consistency between both parameters?

3. do you obtain simillarity between formal and visual evaluation? If yes, more with MOIK or OIF?

4. select 3 best CC from statistical evaluation.

  - EO1 ALI (9 kanałów)*:

1. select the best CC, compare with the CC from Landsat

2. select 3 CC:

- only VIS;

- only IR;

- VNIR

3. test OIF i MOIK on ALI data: 14 bits i 8 bits (stretch with saturation. 1%)

*Notice:

Radiometric resolution of ALI: 14 bits.  ALI channels correspond the Landsat.

- Summary